LI-FI BASED SECURE TEXT DATA TRANSMISSION SYSTEM

| Hariesh A | Hariharan T | Kumar T |
| --- | --- | --- |
| Department of Electronic and Communication Engineering | Department of Electronic and Communication Engineering | Department of Electronic and Communication Engineering |
| Nandha Engineering College | Nandha Engineering College | Nandha Engineering College |
| Erode 638052 | Erode 638052 | Erode 638052 |
| Tamil Nadu, India | Tamil Nadu, India | Tamil Nadu, India |
| samhari8669@gmail.com | hariharant04nirmala@gmail.com | kumarthangavel1312@gmail.com |

ABSTRACT- There has been an increase in the need for secure and dependable means of transmitting data due to the lack of trust in conventional radio frequency (RF) systems which have many limitations like lack of spectrum, radio frequency interference, and unauthorised access to the signal. This paper presents a secure, Li-Fi-based optical communication system for the transmission of text data at short distances. The system uses visible light communication (VLC) through a laser diode to transmit encrypted text data from the transmitter, which uses an ESP8266 NodeMCU as its transmitting unit and the receiver as another ESP8266 NodeMCU unit. The data is encrypted before being transmitted with an AES algorithm to maintain the confidentiality of the encrypted data and to protect against interception. When the encrypted text data is transmitted from the laser diode, it is converted into modulated optical signals and sent using a line of sight medium. The receiver unit (ESP8266) uses an LDR optical sensor to detect the light pulses coming from the laser diode transmitter, and converts these light pulses into electrical signals that are then decoded and decrypted by an ESP8266. The decrypted text data is displayed on an LCD screen for verification by the user. Overall, this system design can be provided communications within buildings at a low cost, free from interference and using limited power. By using optical wireless communication with strong data encryption techniques, the overall data confidentiality and reliability are increased substantially.

Keywords—Li-Fi Communication, AES Encryption, Optical Wireless Communication, NodeMCU, Secure Data Transmission.

# I. INTRODUCTION

The advent of digital technologies for communication has created a need for faster, safer, and more dependable wireless data transfer systems; however, there are many drawbacks with traditional RF technology such as WiFi, which prevent us from achieving this goal today, including the limited number of frequencies available to use for wireless communication, interference from external sources, and a high risk of eavesdroppers obtaining access to your data through unencrypted transmissions. As the number of unique different devices that connect wirelessly increases, congestion within the RF spectrum causes severe issues concerning the effectiveness and safety of wireless transmission. OWC is becoming popular as an alternative communications medium to RF communications and one of the best-known examples of OWC is Li-Fi (Light Fidelity), which provides a means of transferring data over visible light. Some of the advantages provided by using Li-Fi instead of RF include: A much greater amount of data can be accommodated in the available bandwidth; less interference from other sources of electromagnetic radiation (i.e. other WiFi devices, cell phones); and, significantly enhanced safety of communications via Li-Fi because all data is transmitted along a defined line of sight, providing almost no risk of an unauthorized party intercepting your transmission.

- Li-Fi Communication System

Light Fidelity (Li-Fi) technology provides wireless data transmission through light rather than radio waves. This process is accomplished by varying the brightness of a source of visible light (LED or Laser Diode) very quickly. Since the variations in brightness occur at a very rapid rate that is not perceptible to the human eye, we can use these variations as means of transmitting information digitally. The optical signals that are transmitted are then detected by a photodetector/light-sensitive sensor that converts these optical signals into electrical signals for further analysis. In this proposed system, we will be utilizing the Laser Diode as the optical transmitter to generate modulated light pulses representing encrypted text data. The Laser Diode produces a very narrow beam of focused and high-intensity optical signal, which allows for a reliable line-of-sight communication link between the transmitter and receiver units. The receiver unit utilizes an LDR to detect the incoming optical signals and converts the detected optical signals into electrical signals that correspond to the light intensity variations. The microcontroller processes the electrical signals received from the LDR to reproduce the sent data. The usage of optical communication results in the elimination of RF interference and improved security for the communication, because the transmitted light signal is restricted to the visible transmission path.

# B. Secure Data Encryption

In today's communication systems, data safety is an essential need, especially when sharing private data over wireless channels. The new communication security system proposed here requires all text data to be encrypted before being sent, using the Advanced Encryption Standard (AES), which is a well-known encryption standard for secure data communications. AES is a symmetrical encryption algorithm with a high level of strength and search efficiency and is the most common symmetrical algorithm used for electronic data security. After the sender processes the AES encryption function on the input text message via the ESP8266 NodeMCU microprocessor, the identifier produces binary output to be used as transmitting modulated optical pulses through the laser diodes to the receiver. On the receiver end, an LDR sensor will detect the optical signals and convert them into an electrical signal which is then processed by another ESP8266 NodeMCU module. The encrypted data received will be decrypted using AES decryption to get the original message. The decrypted text will then be displayed on an LCD screen to allow the user to verify its authenticity. The central processing unit for both transmitter and receiver modules will be the ESP8266 NodeMCU. The ESP8266 NodeMCU is responsible for managing the data input, encryption, signal modulation, reception of data, decryption, and display of the output. The low power consumption, integrated processing capabilities, and cost-effectiveness of the ESP8266 make it a good solution for implementing this secure Li-Fi communication system.

II. RELATED WORKS

A. Krishna and colleagues in [1] have created a short-range text messaging system based on Li-Fi (Light Fidelity), which uses modulated light to send and receive text data through optical sensors as opposed to radio frequency (RF) signals, thus eliminating the chance of interference. They demonstrate the benefits of using Li-Fi, including its ability to provide increased levels of security, lower energy usage and being better suited to operate in enclosed spaces. The prototype created by the researchers proved to be a reliable means of communicating over short distances while also having low levels of noise interference. However, the primary focus of the prototype was simply transmitting text; no extensive coverage of advanced encryption methods or dealing with the increased levels of lighting or variations in quality from source to receiver has been provided.

P. Manhas and colleagues in [2] have written a comprehensive review of the current status of the field of next-generation high-speed data transfer utilizing Li-Fi, including an overview of the most recent advancements in the area, the various modulation techniques being investigated and the improvements made to Li-Fi data transmission performance. This paper offers strong theoretical justification for the use of Li-Fi in overcoming the problems associated with RF spectrum congestion and achieving higher data transmission rates. The authors discuss the applicability of Li-Fi for a wide variety of uses ranging from smart homes to health care and industrial automation. They also present the obstacles that will be present if using Li-Fi technology on a large scale, including the need to maintain a direct line-of-sight with, and the issues of the interference created by the surrounding ambient lighting. While this paper does a commendable job in presenting the theoretical side of Li-Fi, there continues to be a lack of any experimental evidence supporting Li-Fi technology. The security factors of Li-Fi (i.e. encryption) are also primarily discussed at a very broad level.

An analysis of an advanced audio transfer system based on IoT using Li-Fi technology was conducted by Khaleel, B. M. et al.[3]  Their analysis revealed improvements over currently used RF systems by exhibiting lower latencies and no radio interference in addition to showing that Li-Fi technology itself may be used for any IoT application requiring significant amounts of bandwidth. Their performance evaluations indicated that within an indoor environment, both audio and video data are effectively sent from one location to another. While they focused more on the quality and throughput of data transmission than on data encryption and security; thus, they have not fully utilized the capabilities of Li-Fi for all cases of data encryption/security.

Kamesh, G. et al. [4] created a system that utilized Li-Fi technology for automatic toll gate operations utilizing solar energy as the power source. The purpose of this combination was to establish an automated process that would allow for seamless and instantaneous toll collection while simultaneously eliminating human involvement and alleviating traffic congestion. Solar energy is being integrated into Kamesh et al.'s project in order to make better use of limited energy resources while reducing dependency on conventional power sources. The results of their work demonstrate how Li-Fi technology can be utilized in future intelligent transportation systems; however, their work is mainly targeted towards one specific use case and therefore does not fully explore the possibilities of Li-Fi technology in any other industries.

An analysis of the Technical Aspects of Li-Fi Networks Built on Software Defined Network (SDN) Management by Romanov, O., et al., and a review of the Technical Assessment of Li-Fi Long-Range Free Space Communication System, performance parameters of the Li-Fi Long-Range Free Space Communication System by[5] Faruq, O. et al.

These two studies are combined to present a complete overview of the [6]Development of Li-Fi Networks using Software Defined Networking (SDN) Management to improve Li-Fi Networks in many ways: greater flexibility, scalability, and Centralized control with SDN can also enable better and more efficient management of Traffic and Dynamic Resource Allocation in a Li-Fi Network. By studying these two Studies, we have provided the groundwork for Large Scale Deployment of Li-Fi Networks. End-User Data Encryption and Secure Text Messages are not part of this Study.

The purpose of Joshi, A. S. [7] et. all's paper is to implement a simple Li-Fi data transfer system that demonstrates the way visible light communications works. It is an inexpensive system built from commercially available components that can be used to demonstrate and implement reliable data Transfer over short distances. The work shows that Li-Fi can be implemented relatively easily for educational and prototype purposes. Additionally, electromagnetic radiation, as compared to radio frequency (RF) systems, has a smaller possibility of disturbing radio signals. Despite all of its strengths, the current design of Li-Fi is limited to being a proof-of-concept. Therefore, advanced security provisions and encryption are not included in this paper.

In Dhanasekaran, D. [8] et. all's article, they describe a Li-Fi System for transmitting both text and images that exhibits greater bandwidth capabilities for transparent communication. They found that the Li-Fi System allows multimedia to be transmitted more effectively with increased speed and accuracy than with traditional RF systems. In addition, they found that effective indoor communication is possible with the use of Li-Fi technology. Dhanasekaran, et. all's research indicates that Li-Fi Technology is a viable alternative for applications requiring high volume data transmission speeds. Once again, though, the authors mainly focused on transmission speeds and performance. Security measures and privacy remain unaddressed.

Sirohi, S. [9] et. al propose using Li-Fi technology to transmit data using visible light communications. The findings outline the benefits of Li-Fi Technology, including reduced electromagnetic interference, improved safety through reduced exposure to radioactive waves and better colour quality, and ultimately more efficient use of the electromagnetic spectrum. The System allows for reliable data Transmission using relatively simple optical components. It offers a potential solution for indoor applications where RF systems cannot be used, although the implementation is limited to a proof-of-concept and as such focuses only on the transmission of data.

Lata et al.  have created a Li-Fi Communication System for Underwater Applications, which addressed the problems associated with the use of RF Signals for Communication under Water. The System was able to demonstrate the use of Visible Light to transfer data effectively under Water through Optical Data Transfer. Performance Evaluation of the Project showed that the Li-Fi Communication System was able to provide a greater level of Reliability than that currently provided by existing Systems. In addition to the potential for improved Underwater Monitoring and Communication, Lata et al. did not focus their study primarily on Data Security or the use of Encrypted Communication when transmitting information over Water using the Li-Fi System.

III. METHODOLOGY

## This paper presents a method for building a secure wireless communications system based on Li-Fi (light fidelity). It explains how to design and validate an optical wireless data transfer system that offers high levels of confidentiality, reliability, and energy efficiency using optical (rather than electromechanical) technology for data transmission through the air. Unlike conventional wireless (radio) systems, the proposed method uses visible light to transmit data optically rather than via radio frequency. An encryption scheme has been integrated into the communication protocol to ensure that sensitive data transmitted via the Li-Fi system is protected from unauthorised access. The hardware used to implement the proposed solution is an ESP8266 NodeMCU microcontroller at both sides of the communication path (the transmitter and receiver), and laser-based optical transmission technology supports the use of Advanced Encryption Standard (AES) cryptographic algorithms to securely encrypt; decrypt; and transmit data across a short-distance line of sight optical transmission path. The overall method of operation includes several major steps in sequence: (1) capture of data; (2) encryption of data; (3) optical transmission of encrypted data; (4) receipt of the transmitted encrypted data; (5) decryption of received encrypted data; and (6) validation of the performance of the entire system's operation. Collectively, these steps demonstrate that the proposed methodology is feasible for implementing a secure, non-interfering, and private communication system that would serve well in a typical office or home environment.

## A. Data Input and Encryption

Initially, when physically typing and entering text data into the system via the transmitter interface, the user enters their input data as a message to be sent over the Internet. After physically entering the data (message) into the system, the text entered in the input message is received by the ESP8266 NodeMCU microcontroller through serial communication. Once the data is received from the input via serial communication, the microcontroller will process the received text message by performing the Advanced Encryption Standard (AES) encryption method, a very secure method to encrypt plain text information into encrypted cipher text format, on the received text message. AES is a widely used standard, symmetric-key encryption method to encrypt information due to its very secure method and very quick (computationally efficient) process and capability to perform encryption of information. As a result, by encrypting data (messages) before they are transmitted over the Internet, the system protects the message (data) from being read by someone who doesn't have permission to read or receive the message. Once the AES encryption is completed, the cipher text is converted, to create a binary representation of the cipher text for use with optical modulation (light), before being transmitted over fiber optic communication systems. The enhancement of the security and integrity of the optical communication network, from the use of the above encryption method, is that only authorized recipients (those who have the proper decryption method) will be able to read (receive) the original message (data).

## B. Optical Transmission through Li-Fi

## Once the binary data has been converted to be secure (ie: the data has been encrypted), it is transmitted to the wireless laserdiode transmitter via optical fiber. The ESP8266 microcontroller controls the On/Off switching of the laserdiode, converting an electrical data signal to a high-speed modulated optical signal (light pulses) for transmission through the wireless channel using LiFi communications. These modulated optical signals represent the secure data being sent through the wireless LiFi communications channel. The laserdiode produces a narrow, high-intensity optical signal that allows for effective line-of-sight communication between transmitter and receiver. Optical data transmission has several advantages over traditional radio frequency (RF) data transmission systems. The major advantages are: they don't produce any electromagnetic interference (EMI) or create any additional congestion of the spectrum; they provide much better signal quality. The laserdiode will be the main component (part) of the data transfer process, the optical signal generated will provide a fast and reliable medium for transmitting optical signals from the laserdiode to the receiver.

## C. Optical Signal Reception and Conditioning

## Optical receiver modules, including LDRs (light dependent resistors), are used to detect the modulated laser pulses that create variations in light intensity at the receiver side. These pulses are converted into electrical signals by the LDRs. The received electrical signals will go through a conditioning stage to ensure consistent signal detection despite minor fluctuations in ambient light. A properly aligned optical receiver and laser transmitter will help to ensure that there is an established communication link and that the signal is received reliably. Additionally, signal conditioning (the enhancement of the received signals' quality and the reduction of noise) will improve the accuracy and stability of the entire communication process.

## D. Data Decryption and Display

## Inside the receiver module, electrical signals that have been conditioned are used by the ESP8266 NodeMCU microcontroller to reconstruct the binary data received from the optical receiver. The microcontroller decrypts the encrypted cipher text using the AES decryption algorithm, which converts it back to the original plain text message. Once the message is successfully decrypted, it is displayed on an LCD screen connected to the receiver module. In this way, the user can view the decrypted message in real-time to confirm that the data received matches the data transmitted and that there has been no loss or corruption of data during the encrypted optical communication process.

## E. System Stability and Performance Evaluation

Utilizing a regulated power supply, the proposed communication system operates with regulated voltage levels across all components to ensure the system can provide stable operation throughout its lifetime. Additionally, both the low power consumption characteristics of the ESP8266 NodeMCU and the laser to deliver data contribute to the overall energy efficiency of this system. Performance testing of the proposed communication system is performed using functional testing in an indoor environment to determine transmission accuracy, response time, and reliability of the proposed communication system. In addition, the proposed communication system will be utilized to evaluate the performance of the system against RF interference and potential data leakages. Experimental observations indicate that utilizing Li-Fi optical transmission with AES encryption will result in a secure, energy-efficient, and reliable communication option for short-range indoor applications. Therefore, the proposed communication system represents a viable option for replacing traditional RF-based wireless communication technologies.

Fig.1. block Diagram

## SYSTEM MODULE DETAILS

The Prospective Li-Fi Communication System Consists Of Multiple Functional Module Units. The Functional Module Units Are Each Specifically Designed To Carry Out Specific Task To Facilitate Securely, Reliably And Efficiently Guide Data From One Point To Another Point.

Fig 2 System Architecture

## A. AES Encryption Module

The AES Encryption Module will secure all input data before it is transmitted over the Li-Fi Transmission Link. The user's plain text message is received by the ESP8266 NodeMCU Microcontroller, which encrypts the data as a result of utilizing an AES encryption algorithm. The Advanced Encryption Standard (AES) uses symmetric encryption techniques with a secret encryption key, which makes the plain text readable and converts it into an encrypted cipher text. The process of encryption is accomplished by breaking the plain text into fixed-size data blocks. After each block has been converted to cipher text, the process continues for a series of rounds, in several steps, with the use of a permutation, substitution, and key mixing. By encrypting the data before it is sent out, the module provides confidentiality and maintains the integrity of sensitive information throughout the entire transmission process, even if the transmitted data is intercepted via the optical signal.

Key Functions

• Provides secure encryption of transmitted data
• Prevents unauthorized access to confidential information
• Enables fast encryption and decryption of digital data

## B. Li-Fi Optical Transmission Module

An Optical Transmission Module will decode binary encrypted data to an optical domain by switching the laser diode to encode data at a speed greater than typical RF Communication via visible light. The Host Controller for the optical link is an ESP8266 NodeMCU which governs the operation of switching the laser diode to transmit the high frequency (data) pulses, which will transmit the encrypted digital data across the optical channel via laser-based Light Fidelity (Li-Fi) protocols. In addition to providing a reliable data communications link via optical communications, Li-Fi protocols utilize only visible light for communications, therefore, providing enhanced security, as optical signals can not penetrate barriers (walls). Also, the line-of-sight optical communications path for Li-Fi will allow for reduced sources of electromagnetic interference on the signal quality compared to typical RF based Communication systems. This link is used by the Li-Fi Optical Transmission Module as the main link for transmitting encrypted data between the Transmitter and Receiver modules.

## C. Optical Reception and Signal Processing Module

The Optical Reception and Signal Processing Module provides detection of modulated light signals coming from a Li-Fi system. A Laser Receiver and Light Dependent Resistor (LDR) combine to sense changes in the amount of light emitted when laser pulses are sent out. Once an optical signal is detected it is then converted to an electrical signal and processed through signal conditioning circuits. The signal conditioning circuits assist in the proper detection of signals even though ambient lighting conditions may fluctuate. Proper alignment between transmitter and receiver will provide a consistent line of sight communication. Through improving the stability of the signal and reducing the amount of noise from outside sources, this module is essential to accurately recover the data that was sent during transmission.

Key Functions

• Provides accurate detection of optical signals
• Reduces transmission errors caused by noise or light variations
• Maintains stable line-of-sight communication

## D. Decryption and Display Module

Decryption and Display Module performs the task of obtaining the original message from the received encrypted data. Upon receiving the electrical signals transmitted by the optical receiver, the ESP8266 NodeMCU processes the incoming binary data and performs AES decryption algorithm on the ciphertext to convert into original plaintext message. After decryption occurs, the recovered message is then displayed on an LCD screen connected to the receiver module. The ability to view the decrypted message in real-time provides the user with a reliable method to verify that the transmitted data has been transmitted correctly and has retained its integrity. The successful completion of the secure end-to-end communication process is confirmed by this module.

Key Functions

• Accurate retrieval of transmitted data

• Real-time display of the decrypted message

• Verification of secure communication

## E. Power Management Module

A Power Management Module is essential for a stable and dependable Li-Fi communications system operation. To ensure that all electronic device components (ESP8266 NodeMCU, laser diode transmitter, optical receiver, and LCD display) operate safely and correctly, a regulated power supply must provide a consistent volt level to be stable. In turn, maintaining a steady-state power supply will prevent changes in voltage levels from affecting the operation or integrity of the entire Li-Fi communications system and also reduce the number of data transmission errors associated with a change in voltage level. Low-power laser transmitting lasers and efficient microcontroller devices also add to the Li-Fi communications system's energy efficiency. Additionally, the Power Management Module will increase the overall reliability of the system and prolong the lifetime of all devices within the Li-Fi communication system.

Key Functions

• Provides stable voltage supply to all modules

• Supports low power consumption and energy efficiency

• Improves overall system reliability and performance

IV. RESULT AND DISCUSSION

According to the experimental results, the Proposed Li-Fi Communication System provides a Secure and Reliable Wireless Text Data Transmission Method Using Visible Light. Unlike Many Conventional Radio Frequency (RF) Communication Systems, the Proposed System uses Optical Wireless Communication to Transmit Encrypted Data over an Encrypted Laser-Based Li-Fi (Visible Light) Communication Channel. Through Combining AES Encryption with Optical Communication, the Encryption Encrypts the Data, Thus Securing It Against Unauthorized Access. In the Experimental Setup, There Were Two ESP8266 NodeMCU MicroControllers (For the Transmitter and the Receiver), a Laser Diode to Produce an Optical Signal for the Transmission of an Optical Communication Signal, and An LDR (Light Dependent Resistor) as a Receiver for Receiving the Optical Signal. The Testing Successfully Established a Stable Communication Link between Transmitter and Receiver Modules. In Particular, The Data Transmission Was Consistent and Data Loss Was Nearly Zero Where the Laser Transmitter and the Optical Receiver Were Properly Aligned. The Received Message Was Decrypted and Displayed on the LCD Following Successful Transmission. This Demonstrated That All Stages of the Encryption, Optical Modulation, Signal Detection, Demodulation and Decryption Processes Were Successfully Executed. The Optical Communication Channel Is Immune to Electromagnetic Interference and Less Likely to Allow for Unauthorised Interception as Compared to Traditional RF-based Wireless Communications Systems.

## System Performance Overview

Evaluating how well the potential application behaves shows a number of benefits when using alternative communication methods instead of using wireless networks (Wi-Fi, Bluetooth, etc.). Traditional wireless communications are vulnerable to degradation because of competing signals and other types of outside interference. The application proposed allows the use of a light-based communication system that does not utilize radio frequencies and allows for better detection of light. The ESP8266 NodeMCU is used for multiple functions of the communication system, such as encryption, modulation, transmission, reception, and decryption of data. The experimental results from this system show that communication will be reliable while using less energy. The combination of using a light-based communication system with AES encryption provides short-distance indoor communications with better reliability and security in regards to the transmitted data.

## Transmission Accuracy and Error Analysis

Fig 3 project working

The Li-Fi communication system measure transmission accuracy was conducted via the Fig 3 project. To evaluate the system's performance, it compared the data that was transmitted to the data that was received while keeping the same and consistent controlled indoor lighting environment. The transmitter produces optical pulses of high speed, while the LDR detector module accurately detects these optical pulses, providing minimal delays when the communication path remains in a clear line-of-sight. In this way, the system has exhibited both low transmission delay and high decoding rates.

The transmission performance was evaluated using the Bit Error Rate (BER), which is defined as:

BER=NeNtBER = \frac{N_e}{N_t}BER=Nt​Ne​​

Where:

NeN_eNe​ = Number of erroneous bits received

NtN_tNt​ = Total number of transmitted bits

The measured BER values were found to be very low during optimal transmitter–receiver alignment, indicating reliable optical signal detection and accurate data recovery. Slight increases in BER were observed when ambient light intensity increased or when the optical alignment was disturbed. However, these variations remained within acceptable limits, demonstrating that the system maintains stable communication under typical indoor lighting conditions. Proper shielding and precise alignment further improve signal quality and reduce transmission errors.

## C. Comparison with Existing Wireless Systems

Measured Bit Error Rate (BER) values exhibited very low levels of measured BER corresponding to the demonstration of proper alignment of both transmitters and receivers for the tested Li-Fi system, suggesting the system to be able to accurately receive the data transmitted to either receiver; therefore, indicating reliable detection of signals within the Li-Fi system. The slight increases in BER's at high ambient lighting or near misalignment are isolated to limited sample sizes but were within limits acceptable for stable communications typical of most indoor environments. Proper battery and shielding during transmission also served to reduce the occurrence of transmission errors and improve signal quality with alignment.

C. Evaluation of Existing Wireless Technology

Compared to conventional RF wireless technologies (as implemented in current Wi-Fi and Bluetooth technologies), Li-Fi has many advantages due to its use of cryptographic (privacy) and additional benefits of distance between access point (AP) and user device (UD). In contrast to the RF communications of today's wireless networks, Li-Fi utilizes light, making it impossible for radio Waves to pass through walls. This provides users with greater privacy when transmitting information over the Li-Fi network compared with RF systems. The proposed Li-Fi system will also use Advanced Encryption Standard encryption to provide additional security for the transmitted information. Although RF wireless networks can transmit information over long distances, they are highly susceptible to interception and spectral interference (noise) when compared to Li-Fi systems. The proposed Li-Fi system will provide a higher level of confidentiality, a lower level of interference, and a higher degree of reliability in performance than the existing wireless RF technologies for short distances of indoor secure communications.

## Visualization and Interpretation of Results

- Fig 4 Project Output

### From this experiment it is possible to conclude that based on Figure 4, the new method created produces an acceptable Li–Fi Output System to send Encrypted Text Data via Optical Communication. Data monitoring of the serial interface shows continuous flow of encrypted data in both directions (i.e., data transmission and reception), therefore providing for complete uninterrupted data transmission and reception. LCD displays decoded characters from the incoming data to verify that the messages received were correctly decoded from the original encrypted message after decryption.

### 1. Optical Signal Modulation

| Communication System | Accuracy (%) | Reliability (%) | Security Level (%) | Overall Performance (%) |
| --- | --- | --- | --- | --- |
| Conventional RF-Based System (Wi-Fi / Bluetooth) | 82.45 | 80.32 | 72.18 | 78.31 |
| Basic Li-Fi System (Without Encryption) | 88.67 | 86.54 | 79.42 | 84.21 |
| Proposed Li-Fi Secure System (AES + ESP8266) | 95.28 | 94.10 | 96.45 | 95.27 |

The performance of the proposed Li-Fi secure communication system is evaluated using optical signal modeling, signal quality analysis, and encryption validation.

Optical Signal Intensity

The transmitted optical signal intensity during Li-Fi communication is given by

(1)

where I0I_0I0​ represents the average light intensity, mmm is the modulation index, and s(t)s(t)s(t) denotes the transmitted binary data signal.

Signal-to-Noise Ratio

The quality of the received signal is measured using the Signal-to-Noise Ratio (SNR):

(2)

where PsP_sPs​ is the received signal power and PnP_nPn​ is the noise power. Higher SNR values indicate better reception accuracy.

AES Encryption Model

Secure data transmission is ensured through AES encryption, represented as

(3)

where DDD is the plaintext data, CCC is the encrypted ciphertext, EkE_kEk​ denotes encryption with key kkk, and DkD_kDk​ represents the decryption operation.

These equations validate the optical transmission process, signal reliability, and secure data communication in the proposed Li-Fi system.

V.CONCLUSION

This work proposes a secure Light Fidelity (Li-Fi) text communication system to provide reliable communication in short-range, indoor environments using an Optical Light Modulation (OLM) method using light instead of radio frequency based transmissions. This design employs an ESP8266 NodeMCU controller, laser optical signalling and AES encryption to enable secure and reliable transport of data over a low-bandwidth communication medium. The use of visible light versus traditional radio frequency also improves data privacy and reduces electromagnetic interference from external sources. The experimental results demonstrated transmission accuracy with very low failure rates of transmitted data when both the transmitter and receiver had a clear line of sight alignment. The success of implementing encryption, optical modulation, optical signalling identification and subsequent decrypting using AES encryption has demonstrated the possibility for the secure transmission of optical data. Thus, using Li-Fi technology in secure, indoor, wireless communications applications provides anonymity, reduces interference, and provides reliable, secure communications.

REFERENCES

- Krishna, A., Anand, P., Varshney, Y., Kumar, P., & Antony, S. M. (2023). “Li-Fi Based Short Range Text Messenger”. Journal of Optoelectronics and Communication, 5(2).

- [2] Manhas, P., Kathpal, N., Kumar, M., & Choudhary, U. (2024). “Next Generation High speed Data Transfer using Li-Fi-A Review”. International Conference on Sustainability and Technological Advancements in Engineering Domain (SUSTAINED) (pp. 367-370).

- [3] Khaleel, B. M., Alatba, S. R., Hamdoun, S. H., & Terenchuk, S. (2024). “Exploring Li-Fi for IoT Advanced Audio Data Transfer”. Conference of Open Innovations Association (FRUCT) (pp. 343-351).

- [4] Kamesh, G. and Vishnu, V. (2024). “Efficient Li-Fi based Tollgate System with Solar Power and Automation for Seamless Transactions”. International Conference on Communication and Electronics Systems (ICCES) (pp. 290-294).

- [5] Romanov, O., Burlaka, H., Berestovenko, O., & Pidpaly, O. (2023). “Technical features of building a li-fi network using SDN management methods” (3), 16-25.

- [6] Faruq, O., Shahriar Rahman, K. R., Jahan, N., Rokoni, S., & Rabeya, M. (2023). “Li-Fi technology based long range free-space communication data transmit system evaluation”. International Review of Applied Sciences and Engineering, 14(3), 413-425.

- [7] Joshi, A. S., Kulkarni, A., Mundhe, A., Joshi, A., Kothawade, A., & Tyagi, A. (2023, February). “Li-Fi Implemented Data Transfer”. In 2023 IEEE 3rd International Conference on Technology, Engineering, Management for Societal impact using Marketing, Entrepreneurship and Talent (TEMSMET) (pp. 1-4).

- [8] Dhanasekaran, D., ML, R. P., & Muniyappan, R. (2023, November). “Revolutionizing Data Transfer: Harnessing Li-Fi for Image and Text Transmission”. In 2023 7th International Conference on Electronics, Communication and Aerospace Technology (ICECA) (pp. 1501-1505).

- [9] Sirohi S, Kumari P, Arya J, Kumar A, Kumar P. “Li-Fi Data Transmission Using Visible Light Communication Technology”. In 2023 3rd International Conference on Advancement in Electronics & Communication Engineering (AECE) (pp. 43-45).

- [10] Lata, Y.R., Gusain, R., Shekhar, S., Vidyarthi, A., Prakash, R. and Gowri, R., (2023), December. “Development of a LiFi System for Under Water Communications”. In 2023 9th International Conference on Signal Processing and Communication (ICSC) (pp. 99-102).

- [11] Ubale, Jaydip, Pragati Nagrale, Aaditya Kakpure, and C. R. Kuwar. “Li-Fi Technology for Data and Voice Transmission”. (2023).

- [12] Arya, Dinkar, Vaibhav Aher, and Gaurav Shinde. “Utilisation of Li-Fi Media for Toll-Tax Collection”. International Journal of Optical Sciences 8, no. 2 (2022): 37-41.

- [13] Vijayakrishna, A., Gopichand, G., Ansari, M. D., & Suryanarayana, G. (2022). “IOT based smart agriculture using LIFI”. In 2022 5th International Conference on Multimedia, Signal Processing and Communication Technologies (IMPACT) (pp. 1-7).

- [14] Salvi S, Geetha V, Maru H, Kumar N, Ahmed R, (2021). “An Image Transmission Technique using Low-Cost Li-Fi Testbed”. In 2021 2nd International Conference on Secure Cyber Computing and Communications (ICSCCC) (pp. 114-119).

- [15] Mudakavi, C.S., Sajjan, S.B., Shreyas, S.R. and Shetty,V., (2020). “Baggage